	

	College of Engineering and Computer Science

Mechanical Engineering Department
Mechanical Engineering 370

Thermodynamics

	
	Spring 2007  Number: 11581  Instructor: Larry Caretto


February 1 Homework Problems
1
What is the difference between pound-mass and pound-force?

2
A woman weighs 180 lbf at a location where g = 32.10 ft/s2.  Determine her weight on the moon where g = 5.47 ft/s2.

3
A 3 kg plastic tank that has a volume of 0.2 m3 is filled with liquid water.  Assuming the density of water is 1000 kg/m3, determine the weight of the combined system.

4
Determine the mass and the weight of the air contained in a room whose dimensions are 6 m X 6 m X 8 m.  Assume the density of air is 1.16 kg/m3.

Jacaranda (Engineering) 3333
Mail Code
Phone: 818.677.6448

E-mail: lcaretto@csun.edu
8348
Fax: 818.677.7062


