	

	College of Engineering and Computer Science

Mechanical Engineering Department
Mechanical Engineering 370

Thermodynamics

	
	Fall 2010  Number: 14319  Instructor: Larry Caretto


Solution to Tenth Quiz – Isentropic Efficiency
A pump in a steam power plant has a feed rate of 100 kg/s of water at a temperature and pressure of T1 = 50oC and P1 = 20 kPa.  The pump has negligible heat transfer, an exit pressure, P2 = 8 MPa, and an isentropic efficiency of 87%.  The pump exit is then fed to a steam generator where heat is added to produce steam at P3 = 8 MPa and T3 = 500oC.  Determine (a) the power input to the pump.
The pump is a work input device so the work, w, is the ideal work divided by the isentropic efficiency: w = ws/s.  If we assume that the pump is a steady flow system with negligible changes in kinetic and potential energy the work is given by the following equation: 
[image: image1.wmf](

)

out

in

u

h

h

m

W

-

=

&

&

.  This is valid since we neglect the heat transfer and have only one inlet and one outlet.  For a pump where the specific volume is nearly constant, the isentropic work is given by the following equation: hout,s – hin = vin(Pout – Pin).  Here we see that the inlet is a slightly subcooled liquid so we sue the approximation that, vin = v(T1 = 50oC, P1 = 20 kPa) ( vf(50oC) = 0.001012 m3/kg by eyeball interpolation.
  The inlet enthalpy is similarly approximated as the enthalpy of the saturated liquid at the given temperature.


[image: image2.wmf](

)

kg

kJ

C

C

C

C

kg

kJ

kg

kJ

kg

kJ

h

o

o

o

o

in

71

.

211

01

.

45

50

01

.

45

06

.

60

78

.

191

90

.

251

78

.

191

=

-

-

-

+

=


We find the ideal (isentropic) work as follows.


[image: image3.wmf](

)

(

)

(

)

kW

m

kPa

s

kW

kPa

kPa

kg

m

s

kg

P

P

m

h

h

m

W

s

s

u

8

.

807

1

20

000

,

20

001012

.

0

100

3

3

2

1

,

2

1

,

-

=

×

×

-

=

-

=

-

=

&

&

&


The actual pump work is then found as
[image: image4.wmf]kW

kW

W

W

s

s

u

u

5

.

928

%

87

8

.

807

,

-

=

-

=

=

h

&

&

       
[image: image5.wmf]kW

W

input

5

.

928

=

&


Determine (b) the heat input to the steam generator.
With the same assumptions for the first law as before (steady, one-inlet, one-outlet, negligible changes in kinetic and potential energies) and no useful work in the steam generator, the first law becomes 
[image: image6.wmf](

)

in

out

h

h

m

Q

-

=

&

&

.  We have hout = h3 = h(P3 = 8 MPa and T3 = 500oC) = 3397.6 kJ/kg.  The value of hin = h2 must be found from the actual pump work.


[image: image7.wmf]kg

kJ

s

kW

kJ

s

kg

kW

kg

kJ

m

W

h

h

u

in

00

.

221

1

100

5

.

928

71

.

211

2

=

×

-

-

=

-

=

&

&


We can now find heat transfer in the steam generator.


[image: image8.wmf](

)

kJ

s

MW

kg

kJ

kg

kJ

s

kg

h

h

m

Q

1000

00

.

221

6

.

3397

100

2

3

×

÷

÷

ø

ö

ç

ç

è

æ

-

=

-

=

&

&




[image: image9.wmf]MW

317.7

Q

=

&


Determine (c) the volume flow rate of steam leaving the steam generator.
The mass flow rate and the volume flow rate are related by the specific volume.  Here we want the volume flow rate at the outlet state where v = v(P3 = 8 MPa and T3 = 500oC) = 0.04175 m3/kg.  We thus have 


[image: image10.wmf]kg

m

s

kg

v

m

V

3

3

3

04175

.

0

100

=

=

&

&




[image: image11.wmf]s

m

175

4

V

3

.

=

&


� Actual interpolation gives:


� EMBED Equation.3  ���





Jacaranda (Engineering) 3519
Mail Code
Phone: 818.677.6448

Email: lcaretto@csun.edu
8348
Fax: 818.677.7062


[image: image12.wmf](

)

kg

m

C

C

C

C

kg

m

kg

m

kg

m

v

o

o

o

o

in

3

3

3

3

0010123

.

0

01

.

45

06

.

50

01

.

45

06

.

60

001010

.

0

001017

.

0

001010

.

0

=

-

-

-

+

=

_1351523544.unknown

_1351541661.unknown

_1351542478.unknown

_1351588561.unknown

_1351588576.unknown

_1351542768.unknown

_1351542254.unknown

_1351541440.unknown

_1351522535.unknown

_1351523396.unknown

_1351522692.unknown

_1299913836.unknown

