	

	College of Engineering and Computer Science

Mechanical Engineering Department
Mechanical Engineering 370

Thermodynamics

	
	Fall 2010  Number: 14319  Instructor: Larry Caretto


Solution to Quiz Twelve – Refrigeration and Air-Standard Cycles
[image: image12.emf]V

P

1

2

3 4

5

The dual cycle is an extended model of internal combustion piston engines that generalizes the Otto cycle and the Diesel cycle.  This cycle consists of five steps with air as the working fluid:

1-2) 
An initial isentropic compression from V1 to V2 = V1/CR, where CR is the compression ratio.

2-3)
A constant volume heat addition in which a fraction, , of the high temperature heat transfer is added.

3-4)
A constant pressure heat addition in which the remaining fraction, 1 – , of the high temperature heat transfer is added.

4-5)
An isentropic expansion to the initial volume, V1 = V5.

5-1)
The exhaust process is modeled as a constant volume heat rejection.

Use the attached air tables to compute the efficiency of this cycle for T1 = 540 R, P1 = 10 psia, CR = 8, and qH = (1 – )qh = 400 Btu/lbm.
The total value of qH is 800 Btu/lbm.  To get the efficiency we can either find the work from the three steps that are not constant volume or we can find the heat rejection, which occurs from point 5 to point 1.  In this solution we will do both to show the equivalence of the results.


[image: image1.wmf](

)

(

)

(

)

H

H

L

H

H

q

u

u

q

q

q

u

u

v

v

P

u

u

q

w

5

1

5

4

3

4

4

3

2

1

1

1

-

-

=

-

=

-

+

-

+

-

=

=

®

h

h


Regardless of the approach used, we will have to analyze the entire cycle to obtain properties at intermediate state points to get us to u5.  We start by considering the isentropic expansion from 1 to 2, for which we can use the equation vr(T2) = vr(T1) V2/V1 = vr(T1)/(CR).  From the air tables at T1 = 520 R, we have vr(T1) = 158.59 and u1 = 88.61 Btu/lbm.  At point 2, vr(T2) = vr(T1)/(CR) = 158.59 / 8 = 19.82.  This value is between temperatures of 1160 R and 1200 R.  Interpolation gives


[image: image2.wmf](

)

m

m

m

m

lb

Btu

lb

Btu

lb

Btu

lb

Btu

u

60

.

203

326

.

20

82

.

19

326

.

20

546

.

18

51

.

201

92

.

208

51

.

201

2

=

-

-

-

+

=


For the constant volume heat addition step, w = 0 and the first law gives u3 = u2 + qH = 203.60 Btu/lbm + 400 Btu/lbm = 603.60 Btu/lbm.  Interpolation at this value of u gives.


[image: image3.wmf]R

lb

Btu

lb

Btu

lb

Btu

lb

Btu

R

R

R

T

m

m

m

m

3088

78

.

601

608

.

603

78

.

601

76

.

610

3080

3150

3080

3

=

÷

÷

ø

ö

ç

ç

è

æ

-

-

-

+

=


The constant pressure heat addition step gives the value of h4 = h3+ qH.  We can find h3 = u3 + RT3.  Combining these equations gives the result below.


[image: image4.wmf](

)

m

m

m

m

H

lb

Btu

lb

Btu

R

R

lb

Btu

lb

Btu

q

RT

u

h

29

.

1215

400

3088

06855

.

0

60

.

603

3

3

4

=

+

×

+

=

+

+

=

a


Interpolation at this value of h gives.


[image: image5.wmf](

)

32386

.

0

034

.

1207

29

.

1215

03

.

1207

17

.

1219

33073

.

0

32064

.

0

33073

.

0

4427

034

.

1207

29

.

1215

03

.

1207

17

.

1219

4400

4400

4400

4

4

=

÷

÷

ø

ö

ç

ç

è

æ

-

-

-

+

=

=

÷

÷

ø

ö

ç

ç

è

æ

-

-

-

+

=

m

m

m

m

r

m

m

m

m

lb

Btu

lb

Btu

lb

Btu

lb

Btu

T

v

R

lb

Btu

lb

Btu

lb

Btu

lb

Btu

R

R

R

T


We can compute the isentropic expansion from v4 to v5 using the equation have vr(T5) = vr(T4) v5/v4 , but we have to know the ratio v5/v4 for this calculation.  We can find this ratio from the ideal gas law and the cycle structure which has v3 = v2 = v1/(CR), v5 = v1, and P4 = P3.  This gives the value of v4/v5 as follows

[image: image6.wmf]3

4

5

4

3

4

5

3

4

1

3

4

2

3

4

3

3

3

4

3

4

4

4

4

)

(

)

(

)

(

T

CR

T

v

v

T

CR

T

v

T

CR

T

v

T

T

v

T

T

v

v

RT

RT

P

RT

P

RT

v

=

Þ

=

=

=

=

=

=

=


From the final equation we can find v4/v5 = T4/[(CR)T3 = (4427 R)/[(8)(3088 R)] = 0.17920.  We can then find vr(T5) = vr(T4) v5/v4 0.32386 / 0.17920 = 1.8072.  Interpolation in the tables for this vr value gives.


[image: image7.wmf](

)

m

m

m

m

lb

Btu

lb

Btu

lb

Btu

lb

Btu

u

42

.

503

8899

.

1

8072

.

1

8899

.

1

7999

.

1

37

.

495

13

.

504

37

.

495

5

=

-

-

-

+

=


We can compute the heat rejection as |qL| = u5 – u1 = 503.42 Btu/lbm – 88.61 Btu/lbm = 414.81 Btu/lbm.  The total heat added, |qH| = 400 Btu/lbm + 400 Btu/lbm = 800 Btu/lbm.  So we can finally compute the cycle efficiency.


[image: image8.wmf]m

m

H

L

lb

Btu

lb

Btu

q

q

800

81

.

414

1

1

-

=

-

=

h



n = 48.1%
If use the work to find the efficiency, we have to compute the term P3(4(v4 – v3).  For an ideal gas, P3(4(v4 – v3) = R(T4 – T3) and our work-based efficiency equation becomes

[image: image9.wmf](

)

(

)

(

)

H

H

q

u

u

T

T

R

u

u

q

w

5

4

3

4

2

1

-

+

-

+

-

=

=

h


We have all the required values in this equation except for u4.  We can find this from the values of h4 and T4.  Once u4 is known we have all the information required to find the efficiency.

[image: image10.wmf](

)

m

m

m

lb

Btu

R

R

lb

Btu

lb

Btu

RT

h

u

81

.

911

4427

06855

.

0

29

.

1215

4

4

4

=

×

-

=

+

=



[image: image11.wmf](

)

m

m

m

m

m

m

lb

Btu

lb

Btu

lb

Btu

R

R

R

lb

Btu

lb

Btu

lb

Btu

800

42

.

503

811

.

911

3088

4427

·

06855

.

0

51

.

201

61

.

88

÷

÷

ø

ö

ç

ç

è

æ

-

+

-

+

÷

÷

ø

ö

ç

ç

è

æ

-

=

h


As expected, this gives the same result: n = 48.1%
Note that the work equation has not distinguished between work inputs and work outputs.  Simply using the first law or (Pdv gives work inputs as negative.
Jacaranda (Engineering) 3519
Mail Code
Phone: 818.677.6448

E-mail: lcaretto@csun.edu
8348
Fax: 818.677.7062
Jacaranda (Engineering) 3519
Mail Code
Phone: 818.677.6448

E-mail: lcaretto@csun.edu
8348
Fax: 818.677.7062

_1353387100.unknown

_1353387878.unknown

_1353402169.unknown

_1353402187.unknown

_1353402229.unknown

_1353401218.unknown

_1353387508.unknown

_1353217452.unknown

_1353218592.unknown

_1353218529.unknown

_1353187796.unknown

