Chapter 5, Solution 30.

Air is accelerated in a nozzle from 30 m/s to 180 m/s. The mass flow rate, the exit temperature, and the exit area of the nozzle are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Air is an ideal gas with constant specific heats. 3 Potential energy changes are negligible. 4 The device is adiabatic and thus heat transfer is negligible. 5 There are no work interactions.

Properties The gas constant of air is 0.287 kPa.m3/kg.K (Table A-1). The specific heat of air at the anticipated average temperature of 450 K is cp = 1.02 kJ/kg.C (Table A-2).

Analysis (a) There is only one inlet and one exit, and thus . Using the ideal gas relation, the specific volume and the mass flow rate of air are determined to be

AIR

P2 = 100 kPa

V2 = 180 m/s

P1 = 300 kPa

T1 = 200C

V1 = 30 m/s

A1 = 80 cm2

v1=RT1P1=(0.287  kPa⋅m3/kg⋅K)(473  K)300  kPa=0.4525  m3/kg

m=1v1A1V1=10.4525  m3/kg(0.008  m2)(30  m/s)=0.5304  kg/s

(b) We take nozzle as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=m(h2+V22/2)   (since  {Q≃W≃Δpe≃0)0=h2−h1+V22−V122  0=cp,aveT2−T1+V22−V122

Substituting, 	0=(1.02  kJ/kg⋅K)(T2−200C)+(180  m/s)2−(30  m/s)221  kJ/kg1000  m2/s2

It yields		T2 = 184.6C

(c)  The specific volume of air at the nozzle exit is

v2=RT2P2=(0.287  kPa⋅m3/kg⋅K)(184.6+273  K)100  kPa=1.313  m3/kg

m=1v2A2V2  0.5304  kg/s=11.313  m3/kgA2180  m/s → A2 = 0.00387 m2 = 38.7 cm2

Chapter 5, Solution 41.

Nitrogen is decelerated in a diffuser from 200 m/s to a lower velocity. The exit velocity of nitrogen and the ratio of the inlet-to-exit area are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Nitrogen is an ideal gas with variable specific heats. 3 Potential energy changes are negligible. 4 The device is adiabatic and thus heat transfer is negligible. 5 There are no work interactions.

Properties The molar mass of nitrogen is M = 28 kg/kmol (Table A-1). The enthalpies are (Table A-18)

T1=7°C=280 K   →    {h1=8141 kJ/kmolT2=22°C=295 K  →    {h2=8580 kJ/kmol

Analysis  (a) There is only one inlet and one exit, and thus . We take diffuser as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

N2

1

2

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=m(h2+V22/2)   (since  {Q≃W≃Δpe≃0)0=h2−h1+V22−V122=h2−h1M+V22−V122,

Substituting,

0=8580−8141 kJ/kmol28 kg/kmol+V22−200  m/s221  kJ/kg1000  m2/s2

It yields				       V2 = 93.0 m/s

(b)  The ratio of the inlet to exit area is determined from the conservation of mass relation,

or,	1v2A2V2=1v1A1V1  A1A2=v1v2V2V1=RT1P1RT2P2V2V1A1A2=T1P1T2P2V2V1=280  K/60  kPa93.0  m/s295  K/85  kPa200  m/s=0.625

Chapter 5, Solution 49.

Steam expands in a turbine. The change in kinetic energy, the power output, and the turbine inlet area are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible.

STEAM

m = 12 kg/s

·

P1 = 10 MPa

T1 = 450C

V1 = 80 m/s

P2 = 10 kPa

x2 = 0.92

V2 = 50 m/s

W

·

Properties From the steam tables (Tables A-4 through 6)

P1=10  MPa  v1=0.029782  m3/kgh1=3242.4  kJ/kg

and

P2=10  kPa  h2=hf+x2hfg=191.81+0.92×2392.1=2392.5  kJ/kg

Analysis (a) The change in kinetic energy is determined from

Δke=V22−V122=50  m/s2−(80  m/s)221  kJ/kg1000  m2/s2=−1.95 kJ/kg

(b) There is only one inlet and one exit, and thus . We take the turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

m(h1+V122)=Wout+m(h2+V22/2)   (since  {Q≃Δpe≃0)Wout=−mh2−h1+V22−V122

Then the power output of the turbine is determined by substitution to be

Wout=−(12  kg/s)(2392.5−3242.4−1.95) kJ/kg=10.2  MW

(c)  The inlet area of the turbine is determined from the mass flow rate relation,

m=1v1A1V1  A1=mv1V1=(12  kg/s)(0.029782  m3/kg)80  m/s=0.00447   m2

Chapter 5, Solution 54.

Argon gas expands in a turbine. The exit temperature of the argon for a power output of 250 kW is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible. 4 Argon is an ideal gas with constant specific heats.

Properties The gas constant of Ar is R = 0.2081 kPa.m3/kg.K. The constant pressure specific heat of Ar is cp = 0.5203 kJ/kg·C (Table A-2a)

Analysis There is only one inlet and one exit, and thus . The inlet specific volume of argon and its mass flow rate are

ARGON

A1 = 60 cm2

P1 = 900 kPa

T1 = 450C

V1 = 80 m/s

P2 = 150 kPa

V2 = 150 m/s

250 kW

v1=RT1P1=0.2081  kPa⋅m3/kg⋅K723  K900  kPa=0.167  m3/kg

Thus,

m=1v1A1V1=10.167 m3/kg0.006  m280  m/s=2.874  kgs

We take the turbine as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

m(h1+V122)=Wout+m(h2+V22/2)   (since  {Q≃Δpe≃0)Wout=−mh2−h1+V22−V122

Substituting,

250  kJ/s=−(2.874  kg/s)(0.5203  kJ/kg⋅C)(T2−450C)+(150  m/s)2−(80  m/s)221 kJ/kg1000  m2/s2

It yields		T2 = 267.3C

Chapter 5, Solution 66.

Refrigerant-134a is throttled by a valve. The temperature drop of the refrigerant and specific volume after expansion are to be determined.

P1 = 700 kPa

Sat. liquid

P2 = 160 kPa

R-134a

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 Heat transfer to or from the fluid is negligible. 4 There are no work interactions involved.

Properties The inlet enthalpy of R-134a is, from the refrigerant tables (Tables A-11 through 13),

P1=0.7 MPa  T1=Tsat=26.69Ch1=hf=88.82  kJ/kg

Analysis There is only one inlet and one exit, and thus . We take the throttling valve as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−Eout=ΔEsystemä0  (steady)=0   →    {Ein=Eout  →    {mh1=mh2→   h1=h2

since . Then,

P2=160 kPa  hf=31.21  kJ/kg,Tsat=−15.60°Chg=241.11  kJ/kg

Obviously hf <h2 <hg, thus the refrigerant exists as a saturated mixture at the exit state and thus T2 = Tsat = -15.60°C. Then the temperature drop becomes

ΔT=T2−T1=−15.60−26.69=−42.3°C

The quality at this state is determined from

Thus,	x2=h2−hfhfg=88.82−31.21209.90=0.2745v2=vf+x2vfg=0.0007437+0.2745×(0.12348−0.0007437)=0.0344  m3/kg

Chapter 5, Solution 75.

A hot water stream is mixed with a cold water stream. For a specified mixture temperature, the mass flow rate of cold water is to be determined.

Assumptions 1 Steady operating conditions exist. 2 The mixing chamber is well-insulated so that heat loss to the surroundings is negligible. 3 Changes in the kinetic and potential energies of fluid streams are negligible. 4 Fluid properties are constant. 5 There are no work interactions.

H2O

(P = 250 kPa)

T3 = 42C

T1 = 80C

m1 = 0.5 kg/s

T2 = 20C

m2

·

·

Properties Noting that T < Tsat @ 250 kPa = 127.41°C, the water in all three streams exists as a compressed liquid, which can be approximated as a saturated liquid at the given temperature.  Thus,

h1    hf @ 80C  =	335.02 kJ/kg

h2    hf @ 20C  =	  83.915 kJ/kg

h3    hf @ 42C  =	175.90 kJ/kg

Analysis We take the mixing chamber as the system, which is a control volume. The mass and energy balances for this steady-flow system can be expressed in the rate form as

Mass balance:	min−mout=Δmsystemä0  (steady)=0  m1+m2=m3

Energy balance:

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutm1h1+m2h2=m3h3   (since  {Q=W=Δke≃Δpe≃0)

Combining the two relations and solving for  gives

m1h1+m2h2=m1+m2h3

Substituting, the mass flow rate of cold water stream is determined to be

m2=335.02−175.90 kJ/kg175.90−83.915 kJ/kg0.5  kg/s=0.865   kg/s

Chapter 5, Solution 81.

Refrigerant-134a is to be cooled by air in the condenser. For a specified volume flow rate of air, the mass flow rate of the refrigerant is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 There are no work interactions. 4 Heat loss from the device to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 5 Air is an ideal gas with constant specific heats at room temperature.

Properties The gas constant of air is 0.287 kPa.m3/kg.K (Table A-1). The constant pressure specific heat of air is cp = 1.005 kJ/kg·°C (Table A-2). The enthalpies of the R-134a at the inlet and the exit states are (Tables A-11 through A-13)

AIR

R-134a

3

4

1

2

P3=1  MPa  h3=324.64  kJ/kgP4=1  MPa  h4≃hf @ 30C=93.58  kJ/kg

Analysis The inlet specific volume and the mass flow rate of air are

and	v1=RT1P1=0.287  kPa⋅m3/kg⋅K300  K100  kPa=0.861  m3/kg { dot V1v1600  m3/min0.861  m3/kg

We take the entire heat exchanger as the system, which is a control volume. The mass and energy balances for this steady-flow system can be expressed in the rate form as

Mass balance ( for each fluid stream):

min−mout=Δmsystemä0  (steady)=0 → min=mout → m1=m2=ma and m3=m4=mR

Energy balance (for the entire heat exchanger):

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout m1h1+m3h3=m2h2+m4h4   (since   {Q=W=Δke≃Δpe≃0)

Combining the two,	     mah2−h1=mRh3−h4

Solving for :		mR=h2−h1h3−h4ma≃cpT2−T1h3−h4ma

Substituting,

mR=(1.005  kJ/kg⋅°C)(60−27)°C(324.64−93.58) kJ/kg(696.9  kg/min)=100.0   kg/min

Chapter 5, Solution 88.

Water is heated in a heat exchanger by geothermal water. The rate of heat transfer to the water and the exit temperature of the geothermal water is to be determined.

Assumptions 1 Steady operating conditions exist. 2 The heat exchanger is well-insulated so that heat loss to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid. 3 Changes in the kinetic and potential energies of fluid streams are negligible. 4 Fluid properties are constant.

Properties The specific heats of water and geothermal fluid are given to be 4.18 and 4.31 kJ/kg.C, respectively.

Water

25C

Brine

140C

60C

Analysis We take the cold water tubes as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+mh1=mh2   (since Δke≃Δpe≃0)Qin=mcp(T2−T1)

Then the rate of heat transfer to the cold water in the heat exchanger becomes

Q=[mcp(Tout−Tin)]water=(0.2 kg/s)(4.18 kJ/kg.°C)(60°C−25°C)=29.26 kW

Noting that heat transfer to the cold water is equal to the heat loss from the geothermal water, the outlet temperature of the geothermal water is determined from

Q=[mcp(Tin−Tout)]geot.waterTout=Tin−Qmcp=140°C−29.26 kW(0.3 kg/s)(4.31 kJ/kg.°C)=117.4°C