Chapter 5, Solution 109.

A computer is cooled by a fan blowing air through the case of the computer. The required flow rate of the air and the fraction of the temperature rise of air that is due to heat generated by the fan are to be determined.

Assumptions 1 Steady flow conditions exist. 2 Air is an ideal gas with constant specific heats. 3 The pressure of air is 1 atm. 4 Kinetic and potential energy changes are negligible

Properties   The specific heat of air at room temperature is cp = 1.005 kJ/kg.C (Table A-2).

Analysis   (a) We take the air space in the computer as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutQin+Win+mh1=mh2   (since Δke≃Δpe≃0)Qin+Win=mcp(T2−T1)

Noting that the fan power is 25 W and the 8 PCBs transfer a total of 80 W of heat to air, the mass flow rate of air is determined to be

Qin+Win=mcp(Te−Ti)→m=Qin+Wincp(Te−Ti)=(8×10) W+25 W(1005 J/kg.°C)(10°C)=0.0104 kg/s

(b) The fraction of temperature rise of air that is due to the heat generated by the fan and its motor can be determined from

Q=mcpΔT→ΔT=Qmcp=25 W(0.0104 kg/s)(1005 J/kg.°C)=2.4°Cf=2.4°C10°C=0.24=24%

Chapter 5, Solution 114.

A hair dryer consumes 1200 W of electric power when running. The inlet volume flow rate and the exit velocity of air are to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Air is an ideal gas with constant specific heats at room temperature. 3 Kinetic and potential energy changes are negligible. 4 The power consumed by the fan and the heat losses are negligible.

Properties The gas constant of air is 0.287 kPa.m3/kg.K (Table A-1). The constant pressure specific heat of air at room temperature is cp = 1.005 kJ/kg·K (Table A-2)

Analysis We take the hair dryer as the system, which is a control volume since mass crosses the boundary. There is only one inlet and one exit, and thus . The energy balance for this steady-flow system can be expressed in the rate form as

T2 = 47C

A2 = 60 cm2

P1 = 100 kPa

T1 = 22C

We = 1200 W

·

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutWe,in+mh1=mh2   (since   {Qout≃Δke≃Δpe≃0)We,in=m(h2−h1)=mcp(T2−T1)

Substituting, the mass and volume flow rates of air are determined to be

m=We,incpT2−T1=1.2 kJ/s1.005  kJ/kg⋅C47−22C=0.04776  kg/s

v1=RT1P1=0.287  kPa⋅m3/kg⋅K295  K100  kPa=0.8467  m3/kgV1=mv1=0.04776  kg/s0.8467  m3/kg=0.0404 m3/s

(b) The exit velocity of air is determined from the mass balance  to be

v2=RT2P2=(0.287  kPa⋅m3/kg⋅K)(320 K)(100  kPa)=0.9184  m3/kgm=1v2A2V2  V2=mv2A2=(0.04776  kg/s)(0.9184  m3/kg)60×10−4 m2=7.31   m/s

Chapter 5, Solution 127.

A rigid tank initially contains air at atmospheric conditions. The tank is connected to a supply line, and air is allowed to enter the tank until mechanical equilibrium is established. The mass of air that entered and the amount of heat transfer are to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid at the inlet remains constant. 2 Air is an ideal gas with variable specific heats. 3 Kinetic and potential energies are negligible. 4 There are no work interactions involved. 5 The direction of heat transfer is to the tank (will be verified).

Properties The gas constant of air is 0.287 kPa.m3/kg.K (Table A-1). The properties of air are (Table A-17)

Ti=295 K  hi=295.17 kJ/kg1122

Analysis (a) We take the tank as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:        min−mout=Δmsystem   →    mi=m2−m1

| Energy balance: | Ein−Eoutunderbracealignl Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic, potential, etc. energiesQin+mihi=m2u2−m1u1 (since W≃ke≃pe≃0) |
| --- | --- |

The initial and the final masses in the tank are

Pi = 600 kPa

Ti = 22C

V1 = 2 m3

P1 = 100 kPa

T1 = 22C

Q

·

m1=P1VRT1=(100  kPa)(2  m3)(0.287  kPa⋅m3/kg⋅K)(295  K)=2.362  kgm2=P2VRT2=(600  kPa)(2  m3)(0.287  kPa⋅m3/kg⋅K)(350  K)=11.946  kg

Then from the mass balance,

(b) The heat transfer during this process is determined from

Qin=−mihi+m2u2−m1u1−9.584  kg295.17  kJ/kg+11.946  kg250.02  kJ/kg−2.362  kg210.49  kJ/kg−339  kJ   →   Qout=339 kJ

Discussion The negative sign for heat transfer indicates that the assumed direction is wrong. Therefore, we reversed the direction.

Chapter 5, Solution 137.

A pressure cooker is initially half-filled with liquid water. If the pressure cooker is not to run out of liquid water for 1 h, the highest rate of heat transfer allowed is to be determined.

Assumptions 1 This is an unsteady process since the conditions within the device are changing during the process, but it can be analyzed as a uniform-flow process since the state of fluid leaving the device remains constant.  2 Kinetic and potential energies are negligible. 3 There are no work interactions involved.

Pressure

Cooker

4 L

175 kPa

Properties The properties of water are (Tables A-4 through A-6)

P1=175 kPa   →   vf=0.001057  m3/kg,  vg=1.0037  m3/kg                                 uf=486.82  kJ/kg,  ug=2524.5 kJ/kgP2=175  kPa  v2=vg @ 175  kPa=1.0036  m3/kgu2=ug @ 175  kPa=2524.5  kJ/kgPe=175  kPa  he=hg @ 175  kPa=2700.2  kJ/kg

Analysis We take the cooker as the system, which is a control volume since mass crosses the boundary. Noting that the microscopic energies of flowing and nonflowing fluids are represented by enthalpy h and internal energy u, respectively, the mass and energy balances for this uniform-flow system can be expressed as

Mass balance:        min−mout=Δmsystem   →    me=m1−m2

| Energy balance: | Ein−Eoutunderbracealignl Net energy transfer by heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic, potential, etc. energiesQin−mehe=m2u2−m1u1 (since W≃ke≃pe≃0) |
| --- | --- |

The initial mass, initial internal energy, and final mass in the tank are

m1=mf+mg=Vfvf+Vgvg=0.002  m30.001057  m3/kg+0.002  m31.0036  m3/kg=1.893+0.002=1.895  kgU1=m1u1=mfuf+mgug=1.893486.82+0.0022524.5=926.6  kJm2=Vv2=0.004  m31.0037  m3/kg=0.004  kg

Then from the mass and energy balances,

me=m1−m2=1.895−0.004=1.891  kg

Qin=mehe+m2u2−m1u11.891  kg2700.2  kJ/kg+0.004  kg2524.5  kJ/kg−926.6  kJ=4188  kJ

Thus,

Q=QΔt=4188  kJ3600  s=1.163 kW

Chapter 5, Solution 150.

A long roll of large 1-Mn manganese steel plate is to be quenched in an oil bath at a specified rate. The  mass flow rate of the plate is to be determined.

Steel plate

10 m/min

Assumptions The plate moves through the bath steadily.

Properties The density of steel plate is given to be          = 7854 kg/m3.

Analysis The mass flow rate of the sheet metal through the oil bath is

m=ρV=ρwtV=(7854 kg/m3)(1 m)(0.005 m)(10 m/min)=393 kg/min=6.55 kg/s

Therefore, steel plate can be treated conveniently as a “flowing fluid” in calculations.