Chapter 7, Solution 32.

A rigid tank is initially filled with a saturated mixture of R-134a. Heat is transferred to the tank from a source until the pressure inside rises to a specified value. The entropy change of the refrigerant, entropy change of the source, and the total entropy change for this process are to be determined. 

Assumptions 1 The tank is stationary and thus the kinetic and potential energy changes are zero. 2 There are no work interactions.

Analysis (a)  From the refrigerant tables (Tables A-11 through A-13),

P1=200  kPa  u1=uf+x1ufg=38.28+0.4186.21=112.76  kJ/kgs1=sf+x1sfg=0.15457+0.40.78316=0.4678  kJ/kg⋅Kv1=vf+x1vfg=0.0007533+0.40.099867−0.0007533=0.04040  m3/kgP2=400  kPa x2=v2−vfvfg=0.04040−0.00079070.051201−0.0007907=0.7857u2=uf+x2ufg=63.62+0.7857171.45=198.34  kJ/kgs2=sf+x2sfg=0.24761+0.78570.67929=0.7813  kJ/kg⋅K

The mass of the refrigerant is

m=Vv1=0.5  m30.04040  m3/kg=12.38  kg

Then the entropy change of the refrigerant becomes

ΔSsystem=ms2−s1=12.38  kg0.7813−0.4678 kJ/kg⋅K=3.880 kJ/K

(b) We take the tank as the system. This is a closed system since no mass enters or leaves. Noting that the volume of the system is constant and thus there is no boundary work, the energy balance for this stationary closed system can be expressed as

R-134a

200 kPa

Source

35C

Q

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesQin=ΔU=m(u2−u1)

Substituting,

Qin=mu2−u1=12.38  kg198.34−112.76=1059  kJ

The heat transfer for the source is equal in magnitude but opposite in direction. Therefore,

Qsource, out = - Qtank, in = - 1059 kJ

and

ΔSsource=−Qsource,outTsource=−1059  kJ308  K=−3.439 kJ/K

(c) The total entropy change for this process is

ΔStotal=ΔSsystem+ΔSsource=3.880+−3.439=0.442   kJ/K

m=Vv2=1.2  ft30.5453  ft3/lbm=2.201   lbm

Chapter 7, Solution 34.

An insulated rigid tank contains a saturated liquid-vapor mixture of water at a specified pressure. An electric heater inside is turned on and kept on until all the liquid vaporized. The entropy change of the water during this process is to be determined.

Analysis From the steam tables (Tables A-4 through A-6)

We

H2O

2 kg

100 kPa

P1=100  kPa  v1=vf+x1vfg=0.001+0.251.6941−0.001=0.4243  m3/kgs1=sf+x1sfg=1.3028+0.256.0562=2.8168  kJ/kg⋅Kv2=v12

Then the entropy change of the steam becomes

ΔS=ms2−s1=(2 kg)(6.8649−2.8168)  kJ/kg⋅K=8.10   kJ/K

m=Vv2=1.2  ft30.5453  ft3/lbm=2.201   lbm

Chapter 7, Solution 35.

A rigid tank is divided into two equal parts by a partition. One part is filled with compressed liquid water while the other side is evacuated. The partition is removed and water expands into the entire tank. The entropy change of the water during this process is to be determined.

1.5 kg

compressed liquid

300 kPa

60C

Vacuum

Analysis The properties of the water are (Table A-4)

P1=300  kPa  v1≃vf @ 60°C=0.001017 m3/kgs1=sf @ 60°C=0.8313  kJ/kg⋅K2120.0010173

Then the entropy change of the water becomes

ΔS=ms2−s1=1.5  kg0.7556−0.8313 kJ/kg⋅K=−0.114  kJ/K

m=Vv2=1.2  ft30.5453  ft3/lbm=2.201   lbm

Chapter 7, Solution 38.

An insulated cylinder is initially filled with saturated liquid water at a specified pressure. The water is heated electrically at constant pressure. The entropy change of the water during this process is to be determined.

H2O

150 kPa

Sat. liquid

2200 kJ

Assumptions 1 The kinetic and potential energy changes are negligible.  2 The cylinder is well-insulated and thus heat transfer is negligible. 3 The thermal energy stored in the cylinder itself is negligible. 4 The compression or expansion process is quasi-equilibrium.

Analysis   From the steam tables (Tables A-4 through A-6),

P1=150  kPa  v1=vf @ 150  kPa=0.001053  m3/kgh1=hf @ 150  kPa=467.13  kJ/kgs1=sf @ 150  kPa=1.4337  kJ/kg⋅K

Also,	m=Vv1=0.005  m30.001053  m3/kg=4.75  kg

We take the contents of the cylinder as the system. This is a closed system since no mass enters or leaves. The energy balance for this stationary closed system can be expressed as

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesWe,in−Wb,out=ΔUWe,in=m(h2−h1)

since U + Wb = H during a constant pressure quasi-equilibrium process.  Solving for h2,

h2=h1+We,inm=467.13+2200  kJ4.75  kg=930.33 kJ/kg

Thus,

P2=150  kPax2=h2−hfhfg=930.33−467.132226.0=0.2081s2=sf+x2sfg=1.4337+0.20815.7894=2.6384 kJ/kg⋅K

Then the entropy change of the water becomes

ΔS=ms2−s1=4.75  kg2.6384−1.4337 kJ/kg⋅K=5.72   kJ/K

m=Vv2=1.2  ft30.5453  ft3/lbm=2.201   lbm

Chapter 7, Solution 41.

Saturated Refrigerant-134a vapor at 160 kPa is compressed steadily by an adiabatic compressor. The minimum power input to the compressor is to be determined.

Assumptions 1 This is a steady-flow process since there is no change with time. 2 Kinetic and potential energy changes are negligible. 3 The device is adiabatic and thus heat transfer is negligible.

Analysis The power input to an adiabatic compressor will be a minimum when the compression process is reversible.  For the reversible adiabatic process we have s2 = s1.  From the refrigerant tables (Tables A-11 through A-13),

R-134a

2

1

P1=160  kPa  v1=vg @ 160  kPa=0.12348  m3/kgh1=hg @ 160  kPa=241.11  kJ/kgs1=sg @ 160  kPa=0.9419  kJ/kg⋅KP2=900  kPa  h2=277.06  kJ/kg

Also,

m=V1v1=2  m3/min0.12348  m3/kg=16.20  kg/min=0.27  kg/s

There is only one inlet and one exit, and thus . We take the compressor as the system, which is a control volume since mass crosses the boundary. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemä0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eout

Substituting, the minimum power supplied to the compressor is determined to be

Win=0.27  kg/s277.06−241.11 kJ/kg=9.71   kW

m=Vv2=1.2  ft30.5453  ft3/lbm=2.201   lbm

Chapter 7, Solution 53.

A hot copper block is dropped into water in an insulated tank. The final equilibrium temperature of the tank and the total entropy change are to be determined.

Assumptions 1 Both the water and the copper block are incompressible substances with constant specific heats at room temperature. 2 The system is stationary and thus the kinetic and potential energies are negligible. 3 The tank is well-insulated and thus there is no heat transfer.

Properties The density and specific heat of water at 25C are  = 997 kg/m3 and cp = 4.18 kJ/kg.C. The specific heat of copper at 27C is  cp = 0.386 kJ/kg.C (Table A-3).

Analysis We take the entire contents of the tank, water + copper block, as the system. This is a closed system since no mass crosses the system boundary during the process. The energy balance for this system can be expressed as

WATER

Copper

50 kg

120 L

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energies0=ΔU

or,

ΔUCu+ΔUwater=0

[mc(T2−T1)]Cu+[mc(T2−T1)]water=0

where

mwater=ρV=(997  kg/m3)(0.120  m3)=119.6  kg

Using specific heat values for copper and liquid water at room temperature and substituting,

(50  kg)(0.386  kJ/kg⋅°C)(T2−80)°C+(119.6  kg)(4.18  kJ/kg⋅°C)(T2−25)°C=0

T2 = 27.0C

The entropy generated during this process is determined from

ΔScopper=mcavglnT2T1=50  kg0.386  kJ/kg⋅K ln300.0  K353   K=−3.140  kJ/KΔSwater=mcavglnT2T1=119.6  kg4.18  kJ/kg⋅K ln300.0  K298  K=3.344  kJ/K

Thus,

ΔStotal=ΔScopper+ΔSwater=−3.140+3.344=0.204  kJ/K

Chapter 7, Solution 55.

An aluminum block is brought into contact with an iron block in an insulated enclosure. The final equilibrium temperature and the total entropy change for this process  are to be determined.

Assumptions 1 Both the aluminum and the iron block are incompressible substances with constant specific heats. 2 The system is stationary and thus the kinetic and potential energies are negligible. 3 The system is well-insulated and thus there is no heat transfer.

Properties The specific heat of aluminum at the anticipated average temperature of 450 K is cp = 0.973 kJ/kg.C. The specific heat of iron at room temperature (the only value available in the tables) is  cp = 0.45 kJ/kg.C (Table A-3).

Analysis We take the iron+aluminum blocks as the system, which is a closed system. The energy balance for this system can be expressed as

Iron

20 kg

100C

Aluminum

20 kg

200C

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energies0=ΔU

or,

ΔUalum+ΔUiron=0[mc(T2−T1)]alum+[mc(T2−T1)]iron=0

Substituting,

(20  kg)(0.45  kJ/kg⋅K)(T2−100C)+(20  kg)(0.973 kJ/kg⋅K)(T2−200C)=02

The total entropy change for this process is determined from

ΔSiron=mcavglnT2T1=20  kg0.45  kJ/kg⋅K ln441.4  K373  K=1.515  kJ/KΔSalum=mcavglnT2T1=20  kg0.973  kJ/kg⋅K ln441.4  K473  K=−1.346  kJ/K

Thus,

ΔStotal=ΔSiron+ΔSalum=1.515−1.346=0.169 kJ/K