Chapter 7, Solution 66.

Oxygen gas is compressed from a specified initial state to a specified final state. The entropy change of oxygen during this process is to be determined for the case of constant specific heats.

Assumptions At specified conditions, oxygen can be treated as an ideal gas.

Properties The gas constant and molar mass of oxygen are R = 0.2598 kJ/kg.K and M = 32 kg/kmol (Table A-1).

Analysis The constant volume specific heat of oxygen at the average temperature is (Table A-2)

O2

0.8 m3/kg

25C

Tavg=298+5602=429  K  cv, avg=0.690  kJ/kg⋅K

Thus,

s2−s1=cv, avglnT2T1+RlnV2V10.690  kJ/kg⋅K560  K298  K0.2598  kJ/kg⋅K0.1  m3/kg0.8  m3/kg

Chapter 7, Solution 67.

An insulated tank contains  CO2 gas at a specified pressure and volume. A paddle-wheel in the tank stirs the gas, and the pressure and temperature of CO2 rises. The entropy change of CO2 during this process is to be determined using constant specific heats.

Assumptions At specified conditions, CO2 can be treated as an ideal gas with constant specific heats at room temperature.

CO2

1.5 m3

100 kPa

2.7 kg

Properties The specific heat of CO2 is cv = 0.657 kJ/kg.K (Table A-2).

Analysis Using the ideal gas relation, the entropy change is determined to be

P2VT2=P1VT1  T2T1=P2P1=150  kPa100  kPa=1.5

Thus,

ΔS=ms2−s1=mcv, avglnT2T1+RlnV2V1Ã0=mcv, avglnT2T12.7  kg0.657  kJ/kg⋅K

Chapter 7, Solution 73.

Air is compressed steadily by a 5-kW compressor from one specified state to another specified state.  The rate of entropy change of air is to be determined.

Assumptions At specified conditions, air can be treated as an ideal gas. 2 Air has variable specific heats.

AIR COMPRESSOR

P2 = 600 kPa

T2 = 440 K

P1 = 100 kPa

T1 = 290 K

5 kW

Properties The gas constant of air is R = 0.287 kJ/kg.K  (Table A-1).

Analysis From the air table (Table A-17),

T1=290  Ks1∘=1.66802  kJ/kg⋅KT2=440  K s2∘=2.0887  kJ/kg⋅K

Then the rate of entropy change of air becomes

ΔSsys=ms2∘−s1∘−RlnP2P11.6/60  kg/s

Chapter 7, Solution 75.

Air is compressed in a piston-cylinder device in a reversible and adiabatic manner. The final temperature and the work are to be determined for the cases of constant and variable specific heats.

Assumptions 1 At specified conditions, air can be treated as an ideal gas. 2 The process is given to be reversible and adiabatic, and thus isentropic. Therefore, isentropic relations of ideal gases apply.

Properties The gas constant of air is R = 0.287 kJ/kg.K  (Table A-1).  The specific heat ratio of air at low to moderately high temperatures is k = 1.4 (Table A-2).

Analysis (a) Assuming constant specific heats, the ideal gas isentropic relations give

AIR

Reversible

T2=T1P2P1k−1/k=290  K800  kPa100  kPa0.4/1.4=525.3 K

Then,

Tavg=290+525.3/2=407.7 K  cv, avg=0.727 kJ/kg⋅K

We take the air in the cylinder as the system. The energy balance for this stationary closed system can be expressed as

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesWin=ΔU=m(u2−u1)≃mcv(T2−T1)

Thus,

win=cv, avgT2−T1=0.727  kJ/kg⋅K525.3−290 K=171.1 kJ/kg

(b)  Assuming variable specific heats, the final temperature can be determined using the relative pressure data (Table A-17),

and	T1=290  K  Pr1=1.2311u1=206.91  kJ/kgr2P2P1r1800  kPa100  kPa1.2311T2=522.4 Ku2=376.16  kJ/kg

Then the work input becomes

win=u2−u1=376.16−206.91 kJ/kg=169.25 kJ/kg

Chapter 7, Solution 77.

Helium gas is compressed in a piston-cylinder device in a reversible and adiabatic manner. The final temperature and the work are to be determined for the cases of the process taking place in a piston-cylinder device and a steady-flow compressor.

Assumptions 1 Helium is an ideal gas with constant specific heats. 2 The process is given to be reversible and adiabatic, and thus isentropic. Therefore, isentropic relations of ideal gases apply.

Properties The specific heats and the specific heat ratio of helium are cv = 3.1156 kJ/kg.K, cp = 5.1926 kJ/kg.K,  and  k = 1.667 (Table A-2).

He

Rev.

He

Rev.

2

Analysis  (a) From the ideal gas isentropic relations,

1

T2=T1P2P1k−1/k=303  K450  kPa90  kPa0.667/1.667=576.9   K

(a) We take the air in the cylinder as the system. The energy balance for this stationary closed system can be expressed as

Ein−Eoutunderbracealignl    Net energy transferby heat, work, and mass=ΔEsystemunderbracealignlChange in internal, kinetic,   potential, etc. energiesWin=ΔU=m(u2−u1)≃mcv(T2−T1)

Thus,  	win=cvT2−T1=(3.1156  kJ/kg⋅K)(576.9−303)K=853.4 kJ/kg

(b) If the process takes place in a steady-flow device, the final temperature will remain the same but the work done should be determined from an energy balance on this steady-flow device,

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=EoutWin+mh1=mh2Win=m(h2−h1) ≃mcp(T2−T1)

Thus, 	win=cpT2−T1=(5.1926  kJ/kg⋅K)(576.9−303)K=1422.3   kJ/kg

Chapter 7, Solution 80E.

Air is accelerated in an adiabatic nozzle. Disregarding irreversibilities, the exit velocity of air is to be determined.

Assumptions 1 Air is an ideal gas with variable specific heats. 2 The process is given to be reversible and adiabatic, and thus isentropic. Therefore, isentropic relations of ideal gases apply. 2 The nozzle operates steadily.

Analysis Assuming variable specific heats, the inlet and exit properties are determined to be

AIR

1

2

and	T1=1000  R  Pr1=12.30h1=240.98  Btu/lbmr2P2P1r112  psia60  psia12.30T2=635.9  Rh2=152.11  Btu/lbm

We take the nozzle as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutm(h1+V122)=m(h2+V22/2)h2−h1+V22−V122=0

Therefore,

V2=2h1−h2+V12=2240.98−152.11 Btu/lbm25,037  ft2/s21  Btu/lbm+200  ft/s22119 ft/s

Chapter 7, Solution 81.

Air is accelerated in an nozzle, and some heat is lost in the process. The exit temperature of air and the total entropy change during the process are to be determined.

Assumptions 1 Air is an ideal gas with variable specific heats. 2 The nozzle operates steadily.

Analysis (a) Assuming variable specific heats, the inlet properties are determined to be,

AIR

1

2

3.2 kJ/s

(Table A-17)

We take the nozzle as the system, which is a control volume. The energy balance for this steady-flow system can be expressed in the rate form as

Ein−EoutunderbracealignlRate of net energy transfer by heat, work, and mass=ΔEsystemÃ0  (steady)underbracealignlRate of change in internal, kinetic,         potential, etc. energies=0Ein=Eoutm(h1+V122)=m(h2+V22/2)+Qout0=qout+h2−h1+V22−V122

Therefore,

h2=h1−qout−V22−V122=350.49−3.2−320  m/s2−50  m/s221  kJ/kg1000  m2/s2297.34  kJ/kg

At this h2 value we read, from Table A-17,

(b) The total entropy change is the sum of the entropy changes of the air and of the surroundings, and is determined from

where	Δstotal=Δsair+ΔssurrΔsair=s2∘−s1∘−RlnP2P1=1.6924−1.85708−0.287  kJ/kg⋅K ln85  kPa280  kPa=0.1775  kJ/kg⋅K

and

Thus,	Δssurr=qsurr,inTsurr=3.2  kJ/kg293  K=0.0109  kJ/kg⋅Ktotal