ME 470 Gas Power Cycle HW Solutions           Instructor: Shoeleh Di Julio

Chapter 9, Solution 16.

The four processes of an air-standard cycle are described. The cycle is to be shown on P-v and T-s diagrams, and the maximum temperature in the cycle and the thermal efficiency are to be determined.

Assumptions 1 The air-standard assumptions are applicable. 2 Kinetic and potential energy changes are negligible. 3 Air is an ideal gas with constant specific heats.

Properties The properties of air at room temperature are  cp  = 1.005 kJ/kg.K,  cv  = 0.718 kJ/kg·K,  and  k = 1.4 (Table A-2).

v

P

1

2

4

3

q34

q41

qin

Analysis (b)  From the ideal gas isentropic relations and energy balance,

T2=T1P2P1k−1k=300  K1000  kPa100  kPa0.4/1.4=579.2  K

qin=h3−h2=cpT3−T21.005  kJ/kg⋅KT3−579.2max3

s

T

1

2

4

3

qin

q41

q34

(c)     P3v3T3=P4v4T4  T4=P4P3T3=100  kPa1000  kPa3360  K=336 K

qout=q34,out+q41,out=u3−u4+h4−h1cvT3−T4+cpT4−T10.718  kJ/kg⋅K3360−336K+1.005  kJ/kg⋅K336−300K2212 kJ/kgηth=1−qoutqin=1−2212  kJ/kg2800  kJ/kg=21.0%

Discussion The assumption of constant specific heats at room temperature is not realistic in this case the temperature changes involved are too large.

Chapter 9, Solution 37.

An ideal Otto cycle with air as the working fluid has a compression ratio of 9.5. The highest pressure and temperature in the cycle, the amount of heat transferred, the thermal efficiency, and the mean effective pressure are to be determined.

Assumptions 1 The air-standard assumptions are applicable. 2 Kinetic and potential energy changes are negligible. 3 Air is an ideal gas with constant specific heats.

Properties The properties of air at room temperature are  cp  = 1.005 kJ/kg·K,  cv  = 0.718 kJ/kg·K, R = 0.287 kJ/kg·K, and   k = 1.4  (Table A-2).

v

v

P

4

1

3

2

Qin

Qout

Analysis (a)  Process 1-2: isentropic compression.

T2=T1v1v2k−1=308  K9.50.4=757.9  KP2v2T2=P1v1T1  P2=v1v2T2T1P1=9.5757.9  K308  K100  kPa=2338  kPa

Process 3-4: isentropic expansion.

T3=T4v4v3k−1=800  K9.50.4=1969  K

Process 2-3: v = constant heat addition.

P3v3T3=P2v2T2  P3=T3T2P2=1969  K757.9  K2338  kPa=6072  kPa

(b)	m=P1V1RT1=100  kPa0.0006  m30.287  kPa⋅m3/kg⋅K308  K=6.788×10−4 kg

Qin=mu3−u2=mcvT3−T2=6.788×10−4 kg0.718  kJ/kg⋅K1969−757.9 K=0.590  kJ

(c) Process 4-1: v = constant heat rejection.

Qout=m(u4−u1)=mcvT4−T1=−6.788×10−4 kg0.718  kJ/kg⋅K800−308 K=0.240 kJ

Wnet=Qin−Qout=0.590−0.240=0.350  kJ

ηth=Wnet,outQin=0.350  kJ0.590  kJ=59.4%

(d)	Vmin=V2=VmaxrMEP=Wnet,outV1−V2=Wnet,outV1(1−1r)=0.350  kJ0.0006  m31−1/9.5kPa⋅m3kJ=652  kPa

Chapter 9, Solution 65.

An ideal steady-flow Ericsson engine with air as the working fluid is considered. The maximum pressure in the cycle, the net work output, and the thermal efficiency of the cycle are to be determined.

Assumptions Air is an ideal gas.

Properties The gas constant of air is  R = 0.287 kJ/kg.K  (Table A-1).

Analysis (a)  The entropy change during process 3-4 is

s

T

3

2

qin

qout

4

1

1200 K

300 K

s4−s3=−q34,outT0=−150  kJ/kg300  K=−0.5  kJ/kg⋅K

and 	s4−s3=cplnT4T3Ã0−R lnP4P3−0.287  kJ/kg⋅KlnP4120  kPa=−0.5  kJ/kg⋅K

It yields		P4 = 685.2 kPa

(b)  For reversible cycles, qoutqin=TLTH  qin=THTLqout=1200  K300  K150  kJ/kg=600   kJ/kg

Thus,	wnet,out=qin−qout=600−150=450   kJ/kg

(c) The thermal efficiency of this totally reversible cycle is determined from

ηth=1−TLTH=1−300 K1200 K=75.0%

Chapter 9, Solution 66.

An ideal Stirling engine with helium as the working fluid operates between the specified temperature and pressure limits. The thermal efficiency of the cycle, the amount of heat transfer in the regenerator, and the work output per cycle are to be determined.

Assumptions Helium is an ideal gas with constant specific heats.

Properties The gas constant and the specific heat of helium at room temperature are  R = 2.0769 kJ/kg.K, cv  = 3.1156 kJ/kg.K and  cp  = 5.1926 kJ/kg.K  (Table A-2).

Analysis (a) The thermal efficiency of this totally reversible cycle is determined from

s

T

3

2

qin

qout

4

1

2000 K

300 K

ηth=1−TLTH=1−300  K2000  K=85.0%

(b) The amount of heat transferred in the regenerator is

Qregen=Q41,in=mu1−u4=mcvT1−T40.12  kg3.1156  kJ/kg⋅K2000−300 K635.6  kJ

(c) The net work output is determined from

P3v3T3=P1v1T1  v3v1=T3P1T1P3=300  K3000  kPa2000  K150  kPa=3=v2v1s2−s1=cvlnT2T1Ã0+R lnv2v1=2.0769  kJ/kg⋅Kln3=2.282 kJ/kg⋅KQin=mTHs2−s1=0.12  kg2000  K2.282  kJ/kg⋅K=547.6  kJ