Problem set for the first exam.  Try to complete all these questions.  Solutions.

- Calculate the precision, bias and 95 % accuracy for these two situations:

  - Known concentration is 10 ppm; a meter reads, in 10 successive tests: 10.3; 10.7; 10.2; 10.8; 10.8; 10.5; 10.8; 10.9; 10.7; 10.6 ppm

  - Known pH is 7.0; a pH meter reads, in 10 successive tests: 7.1; 7.0; 6.9; 7.1; 7.2; 7.2; 7.1; 7.1; 7.0; 7.2

- Flow rate calculations: find flow rate in liters per minute for each example.

  - 1000 ml in 42.37 seconds

  - 450 ml in 25.67 seconds

  - 0.0432 cubic feet (cf) in 67.12 seconds

  - 3.997 cubic feet per hour (scfh)

- Find arithmetic mean, arithmetic standard deviation, geometric mean and geometric standard deviation for these numbers: 23, 42, 86, 62, 34, 107, 29, 65, 54, 55

- What is the terminal settling velocity of a 4 micron diameter spherical particle with a density of 3.7 g/cc?

- Pressure and temperature correction to flow: use the formulas printed in week two slides to adjust indicated flow rate for a wet test meter and a rotameter to find actual flow.  In each case, the instrument was calibrated at normal temperature and pressure (760 mm Hg, and 298ºK.

  - Flow indicated is 10 liters per minute, but pressure is 650 mm Hg, temperature is 310ºK.

  - Flow indicated is 2 liters per minute, but pressure is 772 mm Hg, and temperature is 12 ºC

  - Flow indicated is 4 liters per minute, but pressure is 758 mm Hg and temperature is 34 ºC