|  | College of Engineering and Computer Science Computer Science Department Computer Science 106 Computing in Engineering and Science |
| --- | --- |
|  | Spring 2006 Class number: 11672 Instructor: Larry Caretto |

Second Programming Project – Developing Multifunction Programs

Objective

In this project provides an example of advanced program development in which functions are tested individually before being combined into a single program.  In this project you will write functions that compute the sum of an infinite series and solve an equation by trial and error.  You will then combine these two functions into a single program.

Background on the assignment

Infinite series for temperature – The heating or cooling of a solid when the temperature of its exterior is changed is an important problem in materials processing.  The simplest case is for a large slab where multidimensional effects can be ignored.  Here we assume that the lengths in the x and y direction are much greater than the length in the z direction.

We are then interested in the temperature change inside the solid, at any point in the z direction, as a function of time, t.  The coordinate system is chosen so that z = 0 is at the center of the solid and the two ends are at z = L and z = -L.  The rate of temperature change depends on a material property, known as the thermal diffusivity, .

We are interested in the case where the solid is initially at a uniform temperature T0, and the ends (at z = -L and z = L) are set to a different temperature, TB, at the initial time point for the calculations, t = 0.  The subsequent temperature at any point in the z direction at any time, T(z,t), is given by the following infinite series.

T(z,t)−T0TB−T0=1−2n=0∞(−1)nλn e−λn2τcos(λnξ)

This equation for T(z,t) uses the initial temperature, T0, and the boundary temperature, TB, plus the following definitions that involve the terms t, z, L, and  previously defined, and the summation index n.

λn=2n+1π2 ξ=zL τ=αtL2

The first task for this project is to write and test a function that evaluates this infinite series.

Solving equations in one unknown – When we have equations like 3x + 7 = 4x – 3, we can readily rearrange the equation to solve for x.  However, in some more complex equations in a single variable, we cannot solve explicitly for x.  For example, consider the following equation:

x = 3 sin(x) + cos(x)

Here we have one equation in one unknown, x, but we cannot find an explicit solution for x.  Such problems can be solved on the computer using numerical methods.  In these methods we write the equation, in the following form

f(x)=0

All that this form means is that we put all the terms on the left hand side of the equation.  Any equation in a single variable can be written in this form. In the example above we asked for the solution of x = 3 sin(x) + cos(x).  This equation can be written as f(x) = 0 in the two different ways shown below.

(I)    f(x) = x – 3 sin(x) – cos(x) = 0		(II)    f(x) = 3 sin(x) + cos(x) – x = 0

Graphically the solution of f(x) = 0 can be viewed as finding the locations where a plot of f(x) versus x crosses the x axis.  For example, a plot of f(x) = x – 3 sin(x) – cos(x) versus x is shown in the figure below.  We can see that this equation will have roots around x = -2.5, x = -0.5, and x = 2.

Plotting an equation using a graphical calculator or a plot in some computer tool like Excel or Matlab is a good first step in examining the roots of f(x) = 0.  This is especially important in solving engineering problems in which we are usually looking for one answer, but the equation we are solving has multiple roots, as in the figure above.  For equations, such as the one plotted above, the initial guess to the solution will determine the root that you eventually find.

Numerical methods for solving f(x) = 0 are classified as bracketing methods, in which we first find two initial guesses that bracket the root and open methods that can start with only one initial guess.  Bracketing methods require an initial searching process to locate the initial guesses.  For this assignment you will use an open method called the secant method to find the roots of f(x) = 0.

The secant method tries to find the value of x for which f(x) equals zero by using a linear interpolation between the two most recent guesses, x1 and x2. The new root, x3, is found by the following equation.

x3=x2+Δx where Δx=−f(x2)x2−x1f(x2)−f(x1)

In order to ensure a smooth convergence process, the value of the change, x = x3 – x2, may be limited to some maximum relative change in estimated value of x.  This is done by specifying a value gmax such that |x| will be less than gmax times the absolute value of the most recent guess.  The secant algorithm, using this limit on the change in x, is shown in the pseudocode below.

- Start with one initial guess, x1.

- Define a second guess that is proportional to x1.  E.g., x2 = 1.1 x1.

- Compute f1 = f(x1) and f2 = f(x2).

- Repeat the follow steps so long as the solution is not converged and the maximum number of allowed iterations is not exceeded:

  - Compute an preliminary increment in x, x, from the equationΔx=−f2x2−x1f2−f1

  - Limit the change in x, x, relative to |x2| by the if statement below.

  - If |x| is greater than some allowed growth fraction, gmax, times the magnitude of the most recent guess, |x2|

    - adjust x as follows: Δxadjusted=gmaxx2ΔxΔx

  - otherwise

    - Set (x)adjusted = x.

  - This completes the process for limiting the value of x.

  - Compute the new guess, x3, as x2 + (x)adjusted.

  - Compute f3 = f(x3).

  - Define a converged solution if either of the following two conditions holds: (1) the absolute value of (x)adjusted is less than or equal to the desired maximum relative error times the absolute value of x3, or (2) the absolute value of f3 is less than some desired minimum function value.

  - If the solution is not converged do the following tasks

    - Replace the oldest guess (x1) by x2 then replace x2 by the value of x3.  Do the same for the f(x) values.  This requires the following steps, in the order shown:

x1 = x2

f1 = f2

x2 = x3

f2 = f3

  - This completes the tasks that are done if the solution is not converged.  (If the solution is converged, we do not have to do anything here; the algorithm will exit the iteration loop because the solution is converged.)

- This completes the code that is repeated so long as the solution is not converged and the maximum number of allowed iterations is not exceeded.

- At this point you have to determine why the loop was exited.  If the loop was exited because the solution was converged, you can return the solution to the calling program.  (In terms of this algorithm the solution to f(x) = 0 is x3, the most recent value of x.)  If the loop was exited because the maximum number of iterations was exceeded, you should return an error code such as -999.999.

Your second task is to write a function that implements the secant method to find the root of any equation written in the form f(x) = 0.  This secant function should have only one argument, which is an initial guess.  It should use the algorithm above to return the final answer (or an error flag if there is no convergence) in the function name.  In order to apply this function to any equation, your secant function will have to call a second function, f(x), that will evaluate the equation you are trying to solve.  The f(x) function will return the value of the equation for any input value of x.  For this task you can use a value of 100 for the maximum iterations, a value of 10-12 for the maximum allowed relative error, a value of 10-12 for the maximum allowed function value, and a value of 25% for gmax, the maximum allowed growth.

Specific tasks for this project

Task one – evaluate the temperature  Write a function that evaluates the infinite series shown on page one to compute the temperature in a one-dimensional plate at any point in the plate, z, and any time, t.  The overall structure for evaluating the infinite series should be the same as the one you used in exercise, but there are some important changes.  In the infinite series here, you cannot express each new term as a simple factor times the previous term as we did in exercise six.  However, the basic code for summing an infinite series and testing for convergence from exercise six can be used here.  Also, the infinite series used here is only part of the overall solution.  You must first subtract two times the series sum from 1, multiply that result by TB – T0, and add T0 to get the desired temperature

This temperature calculation function, which should have a header like the one shown below, will return the value of the temperature, T(z,t) for input values of z and t.

double plateTemperature( double z, double time );

The program variables for the other parameters in the equation for the temperature should be established as global variables to facilitate the use of this function in task three, below.  The values for the other parameters are shown below.

The uniform initial temperature, T0 = 500oC

The plate is stainless steel with α = 4x10-6 m2/s.

The plate has a half-width, L = 0.1 m.

The edges of the plate are set to a temperature, TB = 50oC at time, t = 0.

Write a program whose main function prepares a two-dimensional table of temperature as a function of z and t.  This program should follow the code you used for calculating a two dimensional table in tasks one and two in exercise six.  It will involve two nested loops and a call to the temperature function, which will actually compute the temperatures.  The table should have time values ranging from 200 s to 3000 s in steps of 200 s and z values ranging from -0.1 m to 0.1 m in steps of 0.02 m.

Import your output file into a spreadsheet program and plot the results.  Use the plot to estimate the time at which the temperature at the centerline (z = 0) reaches 100 oC.  This will provide an initial guess for task three.

Task two – find the roots of f(x) = x – 3 sin(x) – cos(x) = 0  Write a function that uses the pseudocode on page three to implement the secant method and use this function in a program that evaluates all the roots of x – 3sin(x) – cos(x) = 0 as shown in the figure on page two.

Your program for this task should consist of three functions: the secant function from the pseudocode, a function to evaluate f(x), and a main program.  The main program should ask the user for an initial guess and then call the secant function to get the correct root.  The f(x) function for this task is shown below

double f( double x )

{

return x – 3 * sin( x ) – cos( x );

}

This f(x) is a specific example of how we can use the secant function to find the root of any equation that can be written in the form f(x) = 0.  All we have to do is to correctly write the equation so that the value of x we want is found when f(x) = 0.  Notice that in our discussion of the secant method we are using x to represent the unknown in the equation.  The actual equation we are trying to solve can have its unknown variable represented by any other symbol.

You can write the main program to take repeated inputs for the initial guess (following the example used in exercise one, task three) or run the same program three times.  Use three different initial guesses to provide accurate answers to the three roots shown in the graph on page two.

Task three – finding the time at which the plate centerline temperature achieves a certain value – This task uses the functions that you wrote in task one for finding the temperature and the secant method function that you wrote in task two.  In this task we want to apply these functions to determining the time required for the centerline temperature (the temperature at z = 0) to reach a certain value, specified as 100oC.  (The data for , L, T0, and TB are the same as in task one.)  The desired final value of 100oC should be represented as a global variable.

In this task, your main function will simply ask the user for an initial guess for the time, call the secant function, and print the output.  (The plot you make for task one will give you the initial guess.)  The secant function and the temperature function will not have to be changed from the ones you prepared in tasks one and two.  However, you will have to write a new f function for this task.

Recall that the secant function you wrote can solve any equation in one unknown.  You have to write the equation in a form f(x) = 0 where x can be any unknown variable.  We then have to write a function that evaluates the equation – in the f(x) = 0 form – for a value of the unknown, x.  In this task, time is the unknown, so it makes sense to write our function as f(time).  What do we compute in this function?  We have to have some equation that equals zero when the centerline temperature equals the desired temperature of 100 oC.  Since we already have a function that computes the temperature for any input value of spatial coordinate z, and time t, we can simply say that we want this computed temperature equal to the desired temperature or that we want the difference between these two temperatures to be equal to zero.

Think about how you would write the f function for this task before looking at the solution in the next section of this assignment.  The f function shown there accounts for the possibility that the temperature calculation routine may return an error flag.

Approach to task three

Your final program for this task should have the set of functions shown below.  In this code, the temperature calculation function is called from the f(time) function.  The f(time) function is called by the secant function, which has no knowledge of the other problem variables.  Because of this the other variables are entered as global variables as shown below.

// declare global variables for alpha, L, T0, TB and desiredT

double ERROR_FLAG = -999.999            // a global constant

int main()

{

// get initial guess, call secant, and print answer

return EXIT_SUCCESS;

}

double secant ( double x1 )

{

// secant function code from task two

}

double f(double time)

{

double const centerline = 0;

double temperature = plateTemperature( centerline, time );

if( temperature == ERROR_FLAG )

return ERROR_FLAG;

else

return temperature - desiredT;

}

double plateTemperature( double z, double time )

{

// Code to compute T(z,t) from task one

}

The program structure outlined above will work as follows.

- Your main program will get an initial guess for the unknown time from the user.  You can use the graphical results from task one to make a good initial guess.  The main function will then call the secant function, passing the initial guess to that function.

- The secant function will use its trial-and-error algorithm to find the value of the unknown time that reduces the error to the specified limit. (Note that the secant function is using the program variable x for the unknown.  It does not care that the unknown in this case is actually time.  It only cares about the values of the variables that it receives a the initial guess and the f function that it calls.)

- In its computations, the secant function will obtain new successive improvements to its estimate for the unknown.  It passes the value of most recent estimate of the unknown (time) to the function f(time).

- The f(time) function will call the temperature calculation function and, if there is no error, will return the difference between the calculated temperature and the desired temperature to the secant function.

- The exchange of information between the secant function, f function, and temperature function will continue until the calculations in the secant function converge or diverge.

- The secant function will return the correct value or an error code to the main program.

Additional thoughts for task three

The approach outlined above makes use of the existing functions for computing the temperature and using the secant method without the need to make any changes in these functions.  This code reuse is an important approach to programming.  It not only subdivides complex problems into simpler ones, but it makes use of existing code that (presumably) has been well tested and thoroughly debugged.

In preparing for this task you should first convince yourself that we are indeed trying to solve one equation in one unknown.  Granted that it is a complex equation, but if the look at our original equation for temperature, T(z,t), and set z = 0 and T(z,t) = Tdesired, we get the following result.

Tdesired−T0TB−T0=1−2n=0∞(−1)nλn e−λn2τcos(λnξCL)

λn=2n+1π2 ξCL=0 τ=αtL2

If we substitute n, CL, and t into the series expression we see that we have one equation in one unknown, the time, t.  One possible approach to task three would be to rewrite the code that computes the infinite series for temperature to evaluate the following equation.

f(t)=Tdesired−T0TB−T0−1−2n=0∞(−1)nλn e−λn2αtL2cos(λnξCL)=0

This shows that we really do have one equation in one unknown that can be written in the form f(t) = 0.  Rather than recode the existing routine, we choose the approach outlined previously to allow reuse of the already constructed temperature evaluation function without any need to change it.

Note that the individual tasks in this project ask you to prepare your code one function at a time.  This is an effective code development technique.  You write and test individual functions as separate programs.  When you are convinced that they are correct you can put them together into a complete program.

Submission requirements

Due Date:	April 25.

You can prepare your submission file for this project so that it does not duplicate any unchanged function code.  Such a file would include the following items:

- The complete code from task one.

- The output table from task one.

- The plot of the data from task one can be submitted as a separate file or printed output.

- The complete code for task two.

- The screen output giving all answers for task two.

- The main function and the f(time) function for task three.

- The screen output with the initial guess and solution for task three.

If you prefer, you can submit the complete code for task three.  Remember to submit a copy of the graph of the results from task one that you used to obtain the initial guess for finding the answer to task three.

Download the file with the correct answers for this project from the same web location that you used to download these instructions.  Your answers should match the values in that file.

Turn in your printed or email submissions by 11:59 pm on the due date.  Make only one submission, either paper or email.

To do this we use an adjusted x = minΔx, gmaxx2ΔxΔx.  In this equation the factor ΔxΔx is used to give the correct sign to the adjusted x.  For this project you can take gmax = 0.25.

An option for computing the temperature is move the 2 outside the series sum inside the summation operator to make it part of each term.  You can then define the initial series sum as the 1 which is outside the sum and subtract each term in the series from the running sum.  You still have to adjust the result for the right-hand side (RHS) of the equation to give the desired temperature, T = (TB – T0)(RHS) + T0.